Title

British Columbia Missing Glacier Polygons: 1985 and 2021

Authors

Alexandre Bevington1,2, Caleb Mathias2, Maria Kroeger2, Brian Menounos2,3

Affiliations

1 British Columbia Ministry of Forests, Omineca Region

2 University of Northern British Columbia

3 Hakai Institute

Keywords

British Columbia, Glaciers

Description

1985 and 2021 missing glacier polygons of British Columbia derived from satellite imagery.

Specifications table

| Subject | Geographical Information System |
| --- | --- |
| Specific subject area | Glaciers within British Columbia, Canada |
| Type of data | GeoPackage (GPKG) |
| CRS | BC Albers EPSG:3005 |

Project file descriptions

| File | Description |
| --- | --- |
| POLY_2021_S2_POST.gpkg | A post-processed dataset containing all the 2021 missing glaciers, the Bolch and Menounos (2010) 2021 glacier dataset, and the previously missing glaciers at the BC/AK border. |
| POLY_1985_LS5_POST.gpkg | A post-processed dataset containing all the 1985 missing glaciers, the Bolch and Menounos (2010) 1985 glacier dataset, and the previously missing glaciers at the BC/AK border. |

Dataset attribute table

| Column Name | Description |
| --- | --- |
| fid | Automatically generated field ID for each polygon |
| area_km2 | Polygon area (km2) |

Available Glacier Inventories for BC

| Name | Reference Year | Total Area (km2) |
| --- | --- | --- |
| Freshwater Atlas Glaciers | 1980s (Unsure of exact vintage) | 28,822 |
| Bolch and Menounos (2010) | 1985 | 27,638 |
| Bolch and Menounos (2010) | 2005 | 24,809 |
| Bevington and Menounos (2022) | 2020 | 22,340 |
| Bevington and Menounos (2022) update https://zenodo.org/record/5900363 | 2021 | 22,632 |
| Manual edits of Bevington and Menounos (2022) 2021 dataset | 2021 | 22,706 |
| POLY_1985_LS5_POST: missing glacier update of Bolch and Menounos (2010) 1985 dataset | 1985 | 30,454 |
| POLY_2021_S2_POST: missing glacier update of Bevington and Menounos (2022) 2021 dataset | 2021 | 23,884 |

Methodology

Methods described below pertain to identifying missing glaciers from the British Columbia (BC) Freshwater Atlas (FWA) glacier boundaries, and creating a new dataset, in the form of digitized polygons, derived from satellite imagery obtained in 1985-1988 and 2021.

Initial glacier polygons were produced from an automated workflow based on Landsat optical satellite imagery for every year between 1984 and 2021 (Bevington and Menounos, 2022). The automated workflow is suitable for most non debris-covered glaciers, but high mapping errors exist for those that are covered by extensive debris, terminate in lakes, or in areas with perennial snow cover. We previously corrected many of these errors for the 2021 dataset through manual digitization and evaluation of polygons using a Sentinel-2 basemap to minimize mapping errors.

Many glaciers were left undetected utilizing the aforementioned automated workflow. The goal of this project is to create a new dataset which includes missing BC glaciers from both the 1985 and 2021 inventories. Using a semi-automated approach, delineated polygons were removed or modified, and new polygons created where automation failed to detect a glacier. A 1:20,000 grid over BC was created for manual systematic surveying of glaciers.

For the 1985 missing glacier inventory, missing glacier extents were automatically delineated over a 1985-1988 Landsat 5 cloudless summer mosaic using a band ratio (Band 4/Band 5) in Google Earth Engine (GEE) and imported into GIS software (QQIS Version 3.28.0-Firenze). Delineated glacier extents were masked by the existing 1985 British Columbia FWA glacier dataset developed by Bolch and Menounos (2010). Systematic review of delineated glaciers in tandem with the Landsat 5 false colour (R:SWIR2, G:NIR, B:RED) mosaic and high spatial resolution satellite imagery available from various public web mapping services (e.g. “ESRI Satellite” and “Google Satellite”) was completed. Scenes were then surveyed manually for glaciers not delineated by the automated workflow using the BC grid.

As the existing 2021 glacier dataset was created utilizing the same automated workflow as the 1985 dataset, based off Bolch and Menounos (2010)iii, they share the same missing glaciers. After completion of the 1985 missing glacier inventory, the dataset was copied and used as a template for further manual edits of 2021 glacier extents utilizing a 2021 Sentinel-2 cloudless summer false colour (R:SWIR1, G:NIR, B:RED) mosaic. Scenes were then surveyed manually for glaciers not delineated by the automated workflow using the BC grid.

The high-resolution datasets are often of an older vintage (>5 years old) and were used to better identify landforms (e.g., debris-cover, landslides, icebergs in proglacial lakes). Each polygon boundary was manually inspected and corrected or deleted. After the completion of manual edits, shared boundaries were dissolved, all polygons simplified and smoothed, and clipped to the BC political boundaryiii.

Digitizing Workflow

In QGIS, editing the dataset first entails selecting “Toggle Editing” on the target GPKG and opening the “Advanced Digitizing Toolbar”. The GPKG attribute table is then opened so polygons can be systematically inspected sorting by column by ID. Each glacier polygon boundary was examined for geomorphological errors. After iterating over delineated glacier polygon IDs, the BC grid was manually surveyed for any missing glaciers not detected via automation. This process is repeated for each BC grid square. The following rules were followed to ensure consistent edits:

- Delete polygons that do not reflect a true glacier (e.g., small snow patches that appear to be seasonal or perennial which exceeded the minimum area threshold).

- As the spatial resolution of the Sentinel-2 and Landsat 5 imagery is relatively low (20 m and 30 m, respectively), the digitization proceeded at a resolution that is much coarser than extents based on aerial photography (e.g., glacier extents within the TRIM digital products). Based on this low resolution, many features such as small nunataks (exposed bedrock surrounded by a glacier) are not mapped.

- Landslides (i.e., debris in the form of sediment and boulders) present on glaciers are included in the polygons since they overly the glacier. This practice is accepted by the glacier mapping community but differs from some of the gross mapping errors in the TRIM program where non-glacier specialists mapped only clean ice.

- Assume crevasse-covered, hummocky, “pitted”, and debris-covered surfaces outboard visible glacier termini indicate underlying ice still connected to the glacier. At minimum, glacier extents encompass these surfaces.

The British Columbia and Alaska border had missing glaciers that were manually delineated for both datasets. After the initial missing glacier edits for both 1985 and 2021 datasets were completed, a second review was undertaken to correct any erroneously edited glaciers. The approach utilized was to select and manually verify 1985 glaciers that do not intersect with 2021 glaciers. The inverse was done for 2021 glaciers to ensure consistency among both datasets.

After all manual edits were completed, the GPKG was repaired using the “Fix geometries” tool to correct errors in geometry (e.g., self-intersection). The dataset was then imported for edits in ArcGIS Pro (Version 3.0.2). First, the dataset was clipped to the BC provincial boundary polygon using the “Pairwise Clip” tool. The “Dissolve” tool was used to merge shared boundaries between split polygons. Due to spatial errors created when making manual edits, many of the shared borders of the glacier polygons are not perfectly touching and may have minute gaps; therefore, the XY Tolerance was set to 15 m to account for these oversights. Multipart features were not allowed in the output of this tool. The dissolved dataset was then processed with the “Simplify Polygon” tool using the Douglas-Peucker simplifying algorithm at a 10 m spatial tolerance. The minimum area was also set to 30000 m2 to remove polygons smaller than that size. The simplified dataset was then processed with the “Smooth Polygon” tool using the Polynomial Approximation with Exponential Kernel (PAEK) smoothing algorithm at a 50 m tolerance.

Known Issues

The 1985 glacier dataset updated with missing glaciers contains the original, mostly unedited 1985 dataset as presented by Bolch et al. (2010). Some edits were made as delineation errors were encountered during the final review of both datasets; however, no systematic effort for manual correction was attempted in contrast to the manual edits made to the 2021 glacier dataset.

Future Work

- Include glacier names, and icefield names.

- Include glacier type information (debris covered, clean ice, etc.).

- Update every 1-5 years.

- Include terrain and morphometry statistics.

References

Bevington, A.R. and Menounos, B. 2022. Accelerated change in the glaciated environments of western Canada revealed through trend analysis of optical satellite imagery. Remote Sens. Environ. 270, 112862.

Bolch, T., Menounos, B. and Wheate, R. 2010. Landsat-based inventory of glaciers in western Canada, 1985–2005. Remote Sensing of the Environment 114, 127-137.

British Columbia Freshwater Atlas Glacier Polygons 
https://catalogue.data.gov.bc.ca/dataset/freshwater-atlas-glaciers

Bevington and Menounos (2022) dataset with 2021 update. 
https://zenodo.org/record/5900363

iii British Columbia Administrative Boundary Dataset 
https://catalogue.data.gov.bc.ca/dataset/province-of-british-columbia-boundary-terrestrial/